# Juniper Retail Hardware Installation Guide

- Hardware Overview:

  - Juniper Router SSR130

  - Juniper Switch EX4100-48MP

- High-Level Installation Steps:

Note:

- Call NOC Retail Deployment at 1-866-783-9557

- Steps in * must be completed under the guidance of NOC Retail Engineer

- Quick look at the network room and be prepared with a ladder if needed (~5mins)

- Unboxing the new hardware to ensure you have everything needed for the night Section 3 (~ 15 mins)

- Label all the cables connected to the existing Cisco router and switches Section 4 (~30 mins)

- *Unrack the existing Cisco router and switches (~15mins)

- *Rack the new Juniper Router, Switches, and Tripp-Lite PDU (~30mins)

- *Complete the initial WAN and LAN cable connection - Sections 6 and 7 (~15mins)

- *Power up the new Juniper router and switches by connecting them to Tripp-Lite Section 8 (~5 mins)

- *Complete the VCP connection Section 9 (~5 mins)

- *Connect layer 2 wired client connection Sections 10 and 11 (~30 mins)

- *Install, Power and connect new Cradlepoint W1850 Section 12 (~20 mins)

- *Re-label the cables connected to the new Juniper Router and Switches (~30 mins)

- Share the final installation pictures with NOC Retail Deployment

- Packed all the old equipment in the box

- Unboxing:

- Hardware:

  - SSR130(x1) + Rackmount kit

  - EX4100-48MP(x2) – Comes with a Rackmount

  - Triplite(x2)

  - Cradlepoint W1850 (x1)

- Cable:

  - Four Cat6 Ethernet Straight through cables - For connection between Router and Switches

  - Two optical Fiber Cables – Switch Virtual Connect

- SFP:

  - Two SFP-1G-T-C - For connection between Router and Switches

  - Four SFPP-10G-SR-C – Switch Virtual Connect

- Label:

Before starting the work, an onsite technician will be requested to label all the cables connected to the existing Cisco Router and Switches. These will be the temporary labels.

- WAN_Port#

  - Example: WAN_Port0

- Switch#_Port#

  - Example: Switch0_Port40

  - Example: S0_P40

- Juniper Hardware Claim code:

Note: Share the claim codes to the NOC Retail Engineer

- ISP Connection to the Juniper Router

| A-side | SFP A-side | Port A-side | Z-side | SFP Z-side | Port Z-side | Cable type |
| --- | --- | --- | --- | --- | --- | --- |
| Juniper SSR130 | NA | 0 | ISP Modem (Primary) | NA | NA | Cat6 Straight |
| Juniper SSR130 | NA | 1 | ISP Modem (Secondary) | NA | NA | Cat6 Straight |
| Juniper SSR130 | NA | 2 | Cradlepoint | NA | LAN 1 | Cat6 Straight |
| Juniper SSR130 | NA | CONSOLE | Cradlepoint | NA | CONSOLE | Cat6 Rollover |

Primary Broadband

Cradlepoint LAN 1

Cradlepoint Console

Secondary  Broadband

- Connection between Juniper Router and Switches

| A-side | SFP A-side | Port A-side | Z-side | SFP Z-side | Port Z-side | Cable type |
| --- | --- | --- | --- | --- | --- | --- |
| Juniper SSR130 | NA | 4 | Juniper EX4100-48MP (Switch 0) | SFP-1G-T-C | Slot 2 Port 0 | Cat6 Straight |
| Juniper SSR130 | NA | 5 | Juniper EX4100-48MP (Switch 1) | SFP-1G-T-C | Slot 2 Port 0 | Cat6 Straight |
| Juniper SSR130 | NA | 6 | Juniper EX4100-48MP (Switch 0) | NA | MGMT | Cat6 Straight |
| Juniper SSR130 | NA | 7 | Juniper EX4100-48MP (Switch 1) | NA | MGMT | Cat6 Straight |

Juniper EX4100-48MP (Switch 0)

Juniper EX4100-48MP (Switch 1)

Juniper SSR130

- PDU Connection

- Make sure to power up the new Juniper hardware in the below order:

  - Router – PDU A Port 8

  - Switch 0 – PDU A Port 6

  - Switch 1 – PDU B Port 4

- Note: NOC Retail will come later to this section to connect the below hardware:

- Cradlepoint – PDU B Port 8

- Modem 1 – PDU A Port 2

- Modem 2 – PDU B Port 1

- Switches Virtual Chassis (VCP) Connection

| A-side | SFP A-side | Port A-side | Z-side | SFP Z-side | Port Z-side | Cable type |
| --- | --- | --- | --- | --- | --- | --- |
| Juniper EX4100-48MP (Switch 0) | SFPP-10G-SR-C | Slot 1 Port 0 | Juniper EX4100-48MP (Switch 1) | SFPP-10G-SR-C | Slot 1 Port 0 | Optical fiber |
| Juniper EX4100-48MP (Switch 0) | SFPP-10G-SR-C | Slot 1 Port 1 | Juniper EX4100-48MP (Switch 1) | SFPP-10G-SR-C | Slot 1 Port 1 | Optical fiber |

Juniper EX4100-48MP (Switch 0)

Juniper EX4100-48MP (Switch 1)

- Layer 2 – Cisco Switch 1 to Juniper Switch 0

| Cisco Switch 1 (C9300-48P) |  | Juniper Switch 0 (EX4100-48MP) |  |  |
| --- | --- | --- | --- | --- |
| Port | Device | Port | Device | Notes |
| 1 | Cash Drawer 1 | 0 | Cash Drawer 1 |  |  |
| 2 | Cash Drawer 2 | 1 | Cash Drawer 2 |  |  |
| 3 | Wireless AP – 04 | 2 | Wireless AP – 03 | Moved |  |
| 4 | Wireless AP – 05 | 3 | Wireless AP – 04 | Moved |  |
| 5 | Wireless AP - 23 | 4 | Wireless AP - 22 | Moved |  |
| 6 | Burglar Alarm monitor | 5 | Burglar Alarm monitor |  |  |
| 7 | Tyco ADT DVR | 6 | Tyco ADT DVR |  |  |
| 8 | QuEST EMS | 7 | QuEST EMS |  |  |
| 9 | Payment Kiosk #1 | 8 | Payment Kiosk #1 |  |  |
| 10 | Payment Kiosk #2 | 9 | Payment Kiosk #2 |  |  |
| 11 | BCM50 Phone System | 10 | BCM50 Phone System |  |  |
| 12 | Mood Media - StoreMusic | 11 | Mood Media - StoreMusic |  |  |
| 13 | Retail_Training | 12 | Training Station 6 |  |  |
| 14 | Printer 5 | 13 | Printer 5 |  |  |
| 15 | MAC Mini Content Server 1 | 14 | MAC Mini Content Server 1 |  |  |
| 16 | ShopperTrak | 15 | ShopperTrak |  |  |
| 17 | ShopperTrak | 16 | ShopperTrak |  |  |
| 18 | Cartier Coupler 1 | 17 | PDU A | New Hardware |  |
| 19 | Cradlepoint | 18 | Cradlepoint |  |  |
| 20 | IP BTS | 19 |  |  |  |
| 21 | Itunes Analytics (Pervacio) | 20 | Itunes Analytics (Pervacio) |  |  |
| 22 | Training orange jack (live on EIT) | 21 | Training orange jack (live on EIT) |  |  |
| 23 | Itunes Analytics (Pervacio) | 22 | Itunes Analytics (Pervacio) |  |  |
| 24 | UMTS Repeater manager | 23 | UMTS Repeater manager |  |  |
| 25 | Printer 1 (Color) | 24 | Printer 1 (Color) |  |  |
| 26 | Printer 2 | 25 | Printer 2 |  |  |
| 27 | Printer 3 | 26 | Printer 3 |  |  |
| 28 | Printer 4 | 27 | Printer 4 |  |  |
| 29 | Brother Label Printer | 28 | Brother Label Printer |  |  |
| 30 | BOH VoIP/Mgr Desk | 29 | BOH VoIP/Mgr Desk |  |  |
| 31 | BOH VoIP/Mgr Desk | 30 | BOH VoIP/Mgr Desk |  |  |
| 32 | BOH VoIP/Mgr Desk | 31 | BOH VoIP/Mgr Desk |  |  |
| 33 | Cartier Coupler 2 | 32 |  |  |  |
| 34 | Convergent Media Player 1 | 33 | Convergent Media Player 1 |  |  |
| 35 | Convergent Media Player 2 | 34 | Convergent Media Player 2 |  |  |
| 36 | Convergent Media Player 3 | 35 | Convergent Media Player 3 |  |  |
| 37 | Convergent Media Player 4 | 36 | Convergent Media Player 4 |  |  |
| 38 | Convergent Media Player 5 | 37 | Convergent Media Player 5 |  |  |
| 39 | Convergent Media Player 6 | 38 | Convergent Media Player 6 |  |  |
| 40 | Convergent Media Player 7 | 39 | Convergent Media Player 7 |  |  |
| 41 | VoIP/POS system | 40 | VoIP/EIT system |  |  |
| 42 | VoIP/POS system | 41 | VoIP/EIT system |  |  |
| 43 | VoIP/POS system | 42 | VoIP/EIT system |  |  |
| 44 | VoIP/POS system | 43 | VoIP/EIT system |  |  |
| 45 | VoIP/POS system | 44 | VoIP/EIT system |  |  |
| 46 | VoIP/POS system | 45 | VoIP/EIT system |  |  |
| 47 | VoIP/POS system | 46 | VoIP/EIT system |  |  |
| 48 | VoIP/POS system | 47 | VoIP/EIT system |  |  |

- Layer 2 – Cisco Switch 2 to Juniper Switch 1

| Switch 2 (C9300-48P) |  |  | Switch 1 (EX4100-48MP) |  |  |
| --- | --- | --- | --- | --- | --- |
| Port | Device | Port |  | Device | Notes |
| 1 | Cash Drawer 3 | 0 |  | Cash Drawer 3 |  |  |
| 2 | Cash Drawer 4 | 1 |  | Cash Drawer 4 |  |  |
| 3 | Cash Drawer 5 | 2 |  | Cash Drawer 5 |  |  |
| 4 | MAC Mini Content Server 2 | 3 |  | MAC Mini Content Server 2 |  |  |
| 5 | Layer3 TV 1 | 4 |  |  |  |  |
| 6 | Layer3 TV_Convergent Touchscreen Media Player | 5 |  | Convergent Touchscreen Media Player |  |  |
| 7 | Layer 3 TV_Convergent Media Player | 6 |  | Convergent Media Player 9 |  |  |
| 8 | Training Station 1 | 7 |  | Wireless AP - 01 | Moved |  |
| 9 | Training Station 2 | 8 |  | Wireless AP - 02 | Moved |  |
| 10 | Training Station 3 | 9 |  | Wireless AP - 05 | Moved |  |
| 11 | Training Station 4 | 10 |  | Wireless AP - 21 | Moved |  |
| 12 | Training Station 5 | 11 |  | Wireless AP - 23 | Moved |  |
| 13 | Training Station 6 | 12 |  | Future AP ports |  |  |
| 14 | Training Station 7 | 13 |  | Future AP ports |  |  |
| 15 | EPIK | 14 |  | EPIK | New Hardware |  |
| 16 | Device Wizard | 15 |  | Device Wizard |  |  |
| 17 | Device Wizard | 16 |  | Device Wizard |  |  |
| 18 | Additional Device | 17 |  | PDU B | New Hardware |  |
| 19 | Printer 6 | 18 |  |  |  |  |
| 20 | Wireless AP – 01 | 19 |  | Training Station 1 | Moved |  |
| 21 | Wireless AP – 02 | 20 |  | Training Station 2 | Moved |  |
| 22 | Wireless AP – 03 | 21 |  | Training Station 3 | Moved |  |
| 23 | Wireless AP – 21 | 22 |  | Training Station 4 | Moved |  |
| 24 | Wireless AP – 22 | 23 |  | Training Station 5 | Moved |  |
| 25 | Cash Drawer 6 | 24 |  |  |  |  |
| 26 | Cash Drawer 7 | 25 |  |  |  |  |
| 27 | Cash Drawer 8 | 26 |  |  |  |  |
| 28 | Cash Drawer 9 | 27 |  |  |  |  |
| 29 | Cash Drawer 10 | 28 |  |  |  |  |
| 30 | Cash Drawer 11 | 29 |  |  |  |  |
| 31 | Backroom Scala Media Player | 30 |  | Backroom Scala Media Player |  |  |
| 32 | Aruba Controller | 31 |  |  |  |  |
| 33 | Convergent Media Player 8 | 32 |  | Convergent Media Player 8 |  |  |
| 34 | Entouch - HVAC | 33 |  | Entouch - HVAC |  |  |
| 35 | RetailNext | 34 |  | RetailNext |  |  |
| 36 | InVue | 35 |  | InVue |  |  |
| 37 |  | 36 |  |  |  |  |
| 38 |  | 37 |  |  |  |  |
| 39 | PICO Cell | 38 |  | PICO Cell |  |  |
| 40 | PICO Cell | 39 |  | PICO Cell |  |  |
| 41 | POC | 40 |  | POC |  |  |
| 42 | POC | 41 |  | POC |  |  |
| 43 | POC | 42 |  | POC |  |  |
| 44 | POC | 43 |  | POC |  |  |
| 45 | POC | 44 |  | POC |  |  |
| 46 | POC | 45 |  | POC |  |  |
| 47 | POC | 46 |  | POC |  |
| 48 | POC | 47 |  | POC |  |

- Cradlepoint W1850 Connection

Note: Technician will reuse the existing cable connection for new Cradlepoint installation

- Connect Cradlepoint LAN 1 to SSR Router Port 2

- Connect Cradlepoint LAN 2 to EX4100 Switch 0 Port 18

- Connect Cradlepoint CONSOLE to SSR CONSOLE