| Juniper SSR130 |  |  |
| --- | --- | --- |
| Port | Description | Notes |
| Port 0 (GE-0/0/0) - RJ45 | WAN PORT | Primary BB |
| Port 1 (GE-0/0/1)- RJ45 | WAN PORT | Secondary BB |
| Port 2 (GE-0/0/2) - RJ45 | WAN PORT | Cradlepoint (Backup) |
| Port 3 (GE-0/0/3) - RJ45 | Not used |  |
| Port 4 (GE-0/0/4) - RJ45 | Connection to Switch 0 - LAN | Connection to Switch 0 port Ge0/2/0 (Cat6 Ethernet cables straight through) |
| Port 5 (GE-0/0/5) - RJ45 | Connection to Switch 1 - LAN | Connection to Switch 1 port Ge1/2/0 (Cat6 Ethernet cables straight through) |
| Port 6 (GE-0/0/6) - RJ45 | Connection to Switch 0 - MGMT | Connection to Switch 0 port MGMT (Cat6 Ethernet cables straight through) |
| Port 7 (GE-0/0/7)- RJ45 | Connection to Switch 1 - MGMT | Connection to Switch 1 port MGMT (Cat6 Ethernet cables straight through) |
| CONSOLE | Console Port connected to Cradlepoint | Connection to Cradlepoint console (Cat6 - Roll-over cable) |

| Switch 0 (EX4100-48MP) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Port | Device |  |  |  |  |  |  |  |
|  |  | Network segme | VLAN | Trunk | Port Security | IP address | Mask | Gateway |
| 0 | Cash Drawer 1 | PCI | 81 | No | NAC | 10.x.y.178 | 255.255.255.240 | 10.x.y.177 |
| 1 | Cash Drawer 2 | PCI | 81 | No | NAC | 10.x.y.179 | 255.255.255.240 | 10.x.y.177 |
| 2 | Wireless AP - C (AP 3- BACKROOM) | WiFi | 80/100/120 | Yes | No | DHCP | 255.255.255.192 | 10.x.y.1 |
| 3 | Wireless AP - D | WiFi | 80/100/120 | Yes | No | DHCP | 255.255.255.192 | 10.x.y.1 |
| 4 | Wireless AP - Y (AP 5) | WiFi | 80/100/120 | Yes | No | DHCP | 255.255.255.192 | 10.x.y.1 |
| 5 | Burglar Alarm monitor | EIT | 80 | No | NAC | 10.x.y.14 | 255.255.255.192 | 10.x.y.1 |
| 6 | Tyco ADT DVR | Tyco | 60 | No | NAC | 10.x.y.246 | 255.255.255.252 | 10.x.y.245 |
| 7 | QuEST EMS | QUEST | 61 | No | NAC | 10.x.y.242 | 255.255.255.252 | 10.x.y.241 |
| 8 | Payment Kiosk #1 | Portals | 79 | No | NAC | 10.x.y.68 | 255.255.255.192 | 10.x.y.65 |
| 9 | Payment Kiosk #2 | Portals | 79 | No | NAC | 10.x.y.69 | 255.255.255.192 | 10.x.y.65 |
| 10 | BCM50 Phone System | VoIP | 50 | No | No | 10.x.y.194 | 255.255.255.224 | 10.x.y.193 |
| 11 | Mood Media - StoreMusic | Portals | 79 | No | NAC | DHCP | 255.255.255.192 | 10.x.y.65 |
| 12 | Training Station 6 | EIT | 80 | No | NAC | DHCP | 255.255.255.192 | 10.x.y.1 |
| 13 | Printer 5 | EIT | 80 | No | NAC | 10.x.y.12 | 255.255.255.192 | 10.x.y.1 |
| 14 | MAC Mini Content Server 1 (HEALTH CHARGER) | Guest Network | 100 | No | Yes(1) | DHCP | 255.255.252.0 | 192.168.100.1 |
| 15 | ShopperTrak | Portals | 79 | No | NAC | DHCP | 255.255.255.192 | 10.x.y.65 |
| 16 | ShopperTrak | Guest Network | 100 | No | No | DHCP | 255.255.252.0 | 192.168.100.1 |
| 17 | PDU A | EIT | 80 | No | NAC | 10.x.y.4 | 255.255.255.192 | 10.x.y.1 |
| 18 | Cradlepoint | EIT | 80 | No | NAC | 10.x.y.9 | 255.255.255.192 | 10.x.y.1 |
| 19 | Convergent DIU 1 | Guest Network | 100 | No | Yes(1) | DHCP | 255.255.252.0 | 192.168.100.1 |
| 20 | Convergent DIU 2 | Guest Network | 100 | No | Yes(1) | DHCP | 255.255.252.0 | 192.168.100.1 |
| 21 | Convergent DIU 3 | Guest Network | 100 | No | Yes(1) | DHCP | 255.255.252.0 | 192.168.100.1 |
| 22 | Itunes Analytics (Pervacio) | Portals | 79 | No | NAC | DHCP | 255.255.255.192 | 10.x.y.65 |
| 23 | UMTS Repeater manager | EIT | 80 | No | NAC | DHCP | 255.255.255.192 | 10.x.y.1 |
| 24 | Printer 1 (COLOR PRINTER) | EIT | 80 | No | NAC | 10.x.y.15 | 255.255.255.192 | 10.x.y.1 |
| 25 | Printer 2 (BLACK AND WHITE PRINTER) | EIT | 80 | No | NAC | 10.x.y.16 | 255.255.255.192 | 10.x.y.1 |
| 26 | Printer 3 | EIT | 80 | No | NAC | 10.x.y.17 | 255.255.255.192 | 10.x.y.1 |
| 27 | Printer 4 | EIT | 80 | No | NAC | 10.x.y.18 | 255.255.255.192 | 10.x.y.1 |
| 28 | Brother Label Printer | EIT | 80 | No | NAC | 10.x.y.19 | 255.255.255.192 | 10.x.y.1 |
| 29 | BOH VoIP/Mgr Desk (MANAGER PHONE) | EIT/VoIP | 80/50 | No | NAC | DHCP | 255.255.255.192 | 10.x.y.1 |
| 30 | BOH VoIP/Mgr Desk | EIT/VoIP | 80/50 | No | NAC | DHCP | 255.255.255.192 | 10.x.y.1 |
| 31 | BOH VoIP/Mgr Desk | EIT/VoIP | 80/50 | No | NAC | DHCP | 255.255.255.192 | 10.x.y.1 |
| 32 |  |  |  |  |  |  |  |  |
| 33 | Brand Focal (LED) | Portals | 79 | No | NAC | DHCP | 255.255.255.192 | 10.x.y.65 |
| 34 | Header Primary (LED) | Portals | 79 | No | NAC | DHCP | 255.255.255.192 | 10.x.y.65 |
| 35 | Header Secondary (LED) | Portals | 79 | No | NAC | DHCP | 255.255.255.192 | 10.x.y.65 |
| 36 | Header Tertiary (LED) | Portals | 79 | No | NAC | DHCP | 255.255.255.192 | 10.x.y.65 |
| 37 | Vertical Product Screen Primary (APPLE 32") | Portals | 79 | No | NAC | DHCP | 255.255.255.192 | 10.x.y.65 |
| 38 | Vertical Product Screen Secondary (SAMSUNG 32") | Portals | 79 | No | NAC | DHCP | 255.255.255.192 | 10.x.y.65 |
| 39 | Welcome Cloud | Portals | 79 | No | NAC | DHCP | 255.255.255.192 | 10.x.y.65 |
| 40 | VoIP/EIT system- Care PC | EIT/VoIP | 80/50 | No | NAC | DHCP | 255.255.255.192 | 10.x.y.1 |
| 41 | VoIP/EIT system- Care PC | EIT/VoIP | 80/50 | No | NAC | DHCP | 255.255.255.192 | 10.x.y.1 |
| 42 | VoIP/EIT system- FOH phone | EIT/VoIP | 80/50 | No | NAC | DHCP | 255.255.255.192 | 10.x.y.1 |
| 43 | VoIP/EIT system | EIT/VoIP | 80/50 | No | NAC | DHCP | 255.255.255.192 | 10.x.y.1 |
| 44 | VoIP/EIT system | EIT/VoIP | 80/50 | No | NAC | DHCP | 255.255.255.192 | 10.x.y.1 |
| 45 | VoIP/EIT system | EIT/VoIP | 80/50 | No | NAC | DHCP | 255.255.255.192 | 10.x.y.1 |
| 46 | VoIP/EIT system | EIT/VoIP | 80/50 | No | NAC | DHCP | 255.255.255.192 | 10.x.y.1 |
| 47 | VoIP/EIT system | EIT/VoIP | 80/50 | No | NAC | DHCP | 255.255.255.192 | 10.x.y.1 |
| ge0-2-0 | Connection to Router Port 4 |  | 1-1005 | Yes | No | N/A | N/A | N/A |
| ge0-2-1 | Open |  |  |  |  |  |  |  |
| ge0-2-2 | Open |  |  |  |  |  |  |  |
| ge0-2-3 | Open |  |  |  |  |  |  |  |
| xe0-1-0 | VPC - Connection to Switch 0 interface xe1-1-0 |  |  |  |  |  |  |  |
| xe0-1-1 | VPC - Connection to Switch 0 interface xe1-1-1 |  |  |  |  |  |  |  |
| xe0-1-2 | Open |  |  |  |  |  |  |  |
| xe0-1-3 | Open |  |  |  |  |  |  |  |
| MGMT | Connection to Router Port 6 |  | 1-1005 | Yes | No | N/A | N/A | N/A |
| CONSOLE |  |  |  |  |  |  |  |  |
|  | Interface VLAN 80 (Switch) | EIT | 80 |  |  | 10.x.y.6 | 255.255.255.192 | 10.x.y.1 |

| Retail Networks |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Description | Vlan | Subnet | Subnet Mask | Gateway | # of Hosts | Statics | DHCP Scopes | Comments |
| EIT / WiFi | 80 | 10.x.y.0/26 | 255.255.255.192 10.x.y.1 |  | 62 | .1 - .19 | .20 - .62 |  |
| Portals | 79 | 10.x.y.64/26 | 255.255.255.192 | 10.x.y.65 | 62 | .65 - .69 | .70 - .126 |  |
| EIT 2 | 80 | 10.x.y.128/27 | 255.255.255.224 | 10.x.y.129 | 30 | .129 - .133 | .134 - .158 |  |
| Open Space |  | 10.x.y.160/29 | 255.255.255.248 10.x.y.161 |  | 6 |  |  |  |
| Open Space |  | 10.x.y.168/29 | 255.255.255.248 10.x.y.169 |  | 6 |  |  |  |
| PCI Devices - Private | 81 | 10.x.y.176/28 | 255.255.255.240 10.x.y.177 |  | 14 |  |  |  |
| VoIP/BCM50 | 50 | 10.x.y.192/27 | 255.255.255.224 | 10.x.y.193 | 30 | .193 - .195 | .196 - .222 |  |
| Open Space |  | 10.x.y.224/28 | 255.255.255.240 | 10.x.y.225 | 14 |  |  |  |
| QUEST | 61 | 10.x.y.240/30 | 255.255.255.252 | 10.x.y.241 | 2 |  |  |  |
| Tyco | 60 | 10.x.y.244/30 | 255.255.255.252 | 10.x.y.245 | 2 |  |  |  |
| Open Space |  | 10.x.y.248/30 | 255.255.255.252 | 10.x.y.249 | 2 |  |  |  |
| Loopback |  | 10.x.y.254/32 | 255.255.255.255 |  | 1 |  |  |  |

| Guest WiFi |  | 100 192.168.100.0/22 255.255.252.0 |  | 192.168.100.1 |  |  | 1022 100.1 - 100.63 100.64 - 103.254 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| L3TV | 120 | 192.168.120.0/22 | 255.255.252.0 | 192.168.120.1 | 1022 | 120.1 - 120.63 | 120.64 - 120.254 |
| Yellow | 130 | 192.168.130.0/24 | 255.255.255.0 | 192.168.130.1 | 254 | - | 130.2 - 120.254 |

| Switch 1 (EX4100-48MP) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Port | Device |  |  |  |  |  |  |  |
|  |  | Network segment | VLAN | Trunk | Port Security | IP address | Mask | Gateway |
| 0 | Cash Drawer 3 | PCI | 81 | No | NAC | 10.x.y.180 | 255.255.255.240 | 10.x.y.177 |
| 1 | Cash Drawer 4 | PCI | 81 | No | NAC | 10.x.y.181 | 255.255.255.240 | 10.x.y.177 |
| 2 | Cash Drawer 5 | PCI | 81 | No | NAC | 10.x.y.182 | 255.255.255.240 | 10.x.y.177 |
| 3 | MAC Mini Content Server 2 | Guest Network | 100 | No | Yes (1) | DHCP | 255.255.255.192 | 192.168.100.1 |
| 4 | Training Station 1 | EIT | 80 | No | NAC | DHCP | 255.255.255.192 | 10.x.y.1 |
| 5 | Training Station 2 | EIT | 80 | No | NAC | DHCP | 255.255.255.192 | 10.x.y.1 |
| 6 | Training Station 3 | EIT | 80 | No | NAC | DHCP | 255.255.255.192 | 10.x.y.1 |
| 7 | Wireless AP - A (AP 1) | WiFi | 80/100/120 | Yes | No | DHCP | 255.255.255.192 | 10.x.y.1 |
| 8 | Wireless AP - B (AP 2) | WiFi | 80/100/120 | Yes | No | DHCP | 255.255.255.192 | 10.x.y.1 |
| 9 | Wireless AP - E | WiFi | 80/100/120 | Yes | No | DHCP | 255.255.255.192 | 10.x.y.1 |
| 10 | Wireless AP - X (AP 4) | WiFi | 80/100/120 | Yes | No | DHCP | 255.255.255.192 | 10.x.y.1 |
| 11 | Wireless AP - Z | WiFi | 80/100/120 | Yes | No | DHCP | 255.255.255.192 | 10.x.y.1 |
| 12 | Future AP ports | WiFi | 80/100/120 | Yes | No | DHCP | 255.255.255.192 | 10.x.y.1 |
| 13 | Future AP ports | WiFi | 80/100/120 | Yes | No | DHCP | 255.255.255.192 | 10.x.y.1 |
| 14 | EPIK | Guest Network | 130 | No | No | DHCP | 255.255.252.0 | 192.168.100.1 |
| 15 | Device Wizard | Guest Network | 130 | No | Yes (1) | DHCP | 255.255.255.0 | 192.168.130.1 |
| 16 | Device Wizard | Guest Network | 130 | No | Yes (1) | DHCP | 255.255.255.0 | 192.168.130.1 |
| 17 | PDU B | EIT | 80 | No | NAC | 10.x.y.5 | 255.255.255.192 | 10.x.y.1 |
| 18 |  |  |  |  |  |  |  |  |
| 19 | Backroom Scala Media Player | Portals | 79 | No | NAC | DHCP | 255.255.255.192 | 10.x.y.65 |
| 20 | Convergent Touchscreen Media Player (HSI TABLET) Service Display Screen Vertical (TMO SERVICES HSI 55") | Portals | 79 | No | NAC | DHCP | 255.255.255.192 | 10.x.y.65 |
| 21 |  | Portals | 79 | No | NAC | DHCP | 255.255.255.192 | 10.x.y.65 |
| 22 | DFW Primary (EXTERIOR SIDE DFW 1) | Portals | 79 | No | NAC | DHCP | 255.255.255.192 | 10.x.y.65 |
| 23 | DFW Secondary (EXTERIOR SIDE DFW 2) | Portals | 79 | No | NAC | DHCP | 255.255.255.192 | 10.x.y.65 |
| 24 | BDFW Primary (BUILDING SIDE DFW 1) | Portals | 79 | No | NAC | DHCP | 255.255.255.192 | 10.x.y.65 |
| 25 | BDFW Secondary (BUILDING SIDE DFW 2) | Portals | 79 | No | NAC | DHCP | 255.255.255.192 | 10.x.y.65 |
| 26 | Welcome Tablet 1 | Portals | 79 | No | NAC | DHCP | 255.255.255.192 | 10.x.y.65 |
| 27 | Welcome Tablet 2 | Portals | 79 | No | NAC | DHCP | 255.255.255.192 | 10.x.y.65 |
| 28 | Welcome Tablet 3 | Portals | 79 | No | NAC | DHCP | 255.255.255.192 | 10.x.y.65 |
| 29 | Welcome Tablet 4 | Portals | 79 | No | NAC | DHCP | 255.255.255.192 | 10.x.y.65 |
| 30 | Welcome Tablet 5 Vertical Display Screen Primary (BRAND DIGITAL PORTAL 75") | Portals | 79 | No | NAC | DHCP | 255.255.255.192 | 10.x.y.65 |
| 31 | Vertical Display Screen Secondary (BRAND DIGITAL PORTAL 75") | Portals | 79 | No | NAC | DHCP | 255.255.255.192 | 10.x.y.65 |
| 32 |  | Portals | 79 | No | NAC | DHCP | 255.255.255.192 | 10.x.y.65 |
| 33 | Entouch - HVAC | Guest Network | 130 | No | Yes (1) | DHCP | 255.255.255.0 | 192.168.130.1 |
| 34 |  |  |  |  |  |  |  |  |
| 35 | InVue | Guest Network | 100 | No | Yes(3) | DHCP | 255.255.252.0 | 192.168.100.1 |
| 36 | Training Station 4 | EIT | 80 | No | NAC | DHCP | 255.255.255.192 | 10.x.y.1 |
| 37 | Training Station 5 | EIT | 80 | No | NAC | DHCP | 255.255.255.192 | 10.x.y.1 |
| 38 | PICO Cell | Guest Network | 100 | No | Yes (1) | 192.168.100.39 | 255.255.252.0 | 192.168.100.1 |
| 39 | PICO Cell | Guest Network | 100 | No | Yes (1) | 192.168.100.40 | 255.255.252.0 | 192.168.100.1 |
| 40 | POC |  |  |  |  |  |  |  |
| 41 | POC |  |  |  |  |  |  |  |
| 42 | POC |  |  |  |  |  |  |  |
| 43 | POC |  |  |  |  |  |  |  |
| 44 | POC |  |  |  |  |  |  |  |
| 45 | POC |  |  |  |  |  |  |  |
| 46 | POC |  |  |  |  |  |  |  |
| 47 | POC |  |  |  |  |  |  |  |
| ge1-2-0 | Connection to Router Port 5 |  | 1-1005 | Yes | No | N/A | N/A | N/A |
| ge1-2-1 | Open |  |  |  |  |  |  |  |
| ge1-2-2 | Open |  |  |  |  |  |  |  |
| ge1-2-3 | Open |  |  |  |  |  |  |  |
| xe1-1-0 | VPC - Connection to Switch 0 interface xe0-1-0 |  |  |  |  |  |  |  |
| xe1-1-1 | VPC - Connection to Switch 0 interface xe0-1-1 |  |  |  |  |  |  |  |
| xe1-1-2 | Open |  |  |  |  |  |  |  |
| xe1-1-3 | Open |  |  |  |  |  |  |  |
| MGMT | Connection to Router Port 6 |  | 1-1005 | Yes | No | N/A | N/A | N/A |
| CONSOLE |  |  |  |  |  |  |  |  |
|  | Interface VLAN 80 (Switch) | EIT | 80 |  |  | 10.x.y.6 | 255.255.255.192 | 10.x.y.1 |

| Retail Networks |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Description | Vlan | Subnet | Subnet Mask | Gateway | # of Hosts | Statics | DHCP Scopes | Comments |
| EIT / WiFi | 80 | 10.x.y.0/26 | 255.255.255.192 10.x.y.1 |  | 62 | .1 - .19 | .20 - .62 |  |
| Portals | 79 | 10.x.y.64/26 | 255.255.255.192 10.x.y.65 |  | 62 | .65 - .69 | .70 - .126 |  |
| EIT 2 | 80 | 10.x.y.128/27 | 255.255.255.224 10.x.y.129 |  | 30 | .129 - .133 | .134 - .158 |  |
| Open Space |  | 10.x.y.160/29 | 255.255.255.248 10.x.y.161 |  | 6 |  |  |  |
| Open Space |  | 10.x.y.168/29 | 255.255.255.248 10.x.y.169 |  | 6 |  |  |  |
| PCI Devices - Private | 81 | 10.x.y.176/28 | 255.255.255.240 | 10.x.y.177 | 14 |  |  |  |
| VoIP/BCM50 | 50 | 10.x.y.192/27 | 255.255.255.224 10.x.y.193 |  | 30 | .193 - .195 | .196 - .222 |  |
| Open Space |  | 10.x.y.224/28 | 255.255.255.240 | 10.x.y.225 | 14 |  |  |  |
| QUEST | 61 | 10.x.y.240/30 | 255.255.255.252 10.x.y.241 |  | 2 |  |  |  |
| Tyco | 60 | 10.x.y.244/30 | 255.255.255.252 10.x.y.245 |  | 2 |  |  |  |
| Open Space |  | 10.x.y.248/30 | 255.255.255.252 10.x.y.249 |  | 2 |  |  |  |
| Loopback |  | 10.x.y.254/32 | 255.255.255.255 |  | 1 |  |  |  |

| Guest WiFi |  | 100 192.168.100.0/22 255.255.252.0 |  | 192.168.100.1 |  |  | 1022 100.1 - 100.63 100.64 - 103.254 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| L3TV |  | 120 192.168.120.0/22 255.255.252.0 |  | 192.168.120.1 |  |  | 1022 120.1 - 120.63 120.64 - 120.254 |
| Yellow | 130 | 192.168.130.0/24 255.255.255.0 |  | 192.168.130.1 | 254 | - | 130.2 - 120.254 |

| Tripp-Lite |  |  |  |
| --- | --- | --- | --- |
| PDU - A |  | PDU - B |  |
| Port | Device | Port | Device |
| 1 |  | 1 | Modem 2 |
| 2 | Modem 1 | 2 |  |
| 3 |  | 3 |  |
| 4 |  | 4 | Switch 1 |
| 5 |  | 5 |  |
| 6 | Switch 0 | 6 |  |
| 7 |  | 7 |  |
| 8 | Router | 8 | Cradlepoint |

EIT - Vlan 80 EIT 2 - Vlan 80 PCI / Cash Drawer - Vlan 81 Portals / Vendor - Vlan 79 TYCO VPN - Vlan 60 Guest Network - Vlan 100 L3TV - Vlan 120

| IP Address Device / Application Router / Switch Port |  |  |  |
| --- | --- | --- | --- |
| 10.x.y.1 | Interface VLAN 80 (Router) | Router | N/A |
| 10.x.y.2 |  |  |  |
| 10.x.y.3 |  |  |  |
| 10.x.y.4 | PDU A | Switch 0 | 17 |
| 10.x.y.5 | PDU B | Switch 1 | 17 |
| 10.x.y.6 | Interface VLAN 80 (Switch 1) | Switch 0 | N/A |
| 10.x.y.7 |  |  |  |
| 10.x.y.8 |  |  |  |
| 10.x.y.9 | Craddlepoint | Switch 0 | 18 |
| 10.x.y.10 |  |  |  |
| 10.x.y.11 |  |  |  |
| 10.x.y.12 | Printer 5 | Switch 0 | 13 |
| 10.x.y.13 |  |  |  |
| 10.x.y.14 | Burglar Alarm (BA) | Switch 0 | 5 |
| 10.x.y.15 | Printer 1 (Color) | Switch 0 | 24 |
| 10.x.y.16 | Printer 2 | Switch 0 | 25 |
| 10.x.y.17 | Printer 3 | Switch 0 | 26 |
| 10.x.y.18 | Printer 4 | Switch 0 | 27 |
| 10.x.y.19 | Brother Label Printer | Switch 0 | 28 |

| IP Address Device / Application Router / Switch Port |  |  |  |
| --- | --- | --- | --- |
| 10.x.y.129 | Interface VLAN 80 (Router) secondary | Router | N/A |
| 10.x.y.130 |  |  |  |
| 10.x.y.131 |  |  |  |
| 10.x.y.132 |  |  |  |
| 10.x.y.133 |  |  |  |

| IP Address Device / Application Router / Switch Port |  |  |  |
| --- | --- | --- | --- |
| 10.x.y.177 | Interface VLAN 81 (Router) | Rotuer | N/A |
| 10.x.y.178 | Cash Drawer 1 | Switch 1 | 49 |
| 10.x.y.179 | Cash Drawer 2 | Switch 2 | 50 |
| 10.x.y.180 | Cash Drawer 3 | Switch 2 | 1 |
| 10.x.y.181 | Cash Drawer 4 | Switch 2 | 2 |
| 10.x.y.182 | Cash Drawer 5 | Switch 2 | 3 |
| 10.x.y.183 | Cash Drawer 6 | Switch 2 | 25 |
| 10.x.y.184 | Cash Drawer 7 | Switch 2 | 26 |
| 10.x.y.185 | Cash Drawer 8 | Switch 2 | 27 |
| 10.x.y.186 | Cash Drawer 9 | Switch 2 | 28 |
| 10.x.y.187 | Cash Drawer 10 | Switch 2 | 29 |
| 10.x.y.188 | Cash Drawer 11 | Switch 2 | 30 |
| 10.x.y.189 |  |  |  |
| 10.x.y.190 |  |  |  |

| IP Address Device / Application Router / Switch Port |  |  |  |
| --- | --- | --- | --- |
| 10.x.y.65 | Interface VLAN 79 (Router) | Router | N/A |
| 10.x.y.66 | DeepNorth | Switch 2 (POC) | 41 |
| 10.x.y.67 |  |  |  |
| 10.x.y.68 | Payment Kiosk #1 | Switch 1 | 9 |
| 10.x.y.69 | Payment Kiosk #2 | Switch1 | 10 |

| IP Address Device / Application Router / Switch Port |  |  |  |
| --- | --- | --- | --- |
| 10.x.y.245 | Interface VLAN 60 (Router) | Rotuer | N/A |
| 10.x.y.246 | Tyco DVR | Switch 1 | 7 |

| IP Address Device / Application Router / Switch Port Quest - Vlan 61 |  |  |  |
| --- | --- | --- | --- |
| 10.x.y.241 | Interface VLAN 61 (Router) | Rotuer | N/A |
| 10.x.y.242 | QuEST EMS | Switch 1 | 8 |

POC POC POC POC POC POC

| IP Address Device / Application Router / Switch POC List Port |  |  |  |  |
| --- | --- | --- | --- | --- |
| 192.168.100.1 | Guest Access (Router) | Rotuer |  | N/A |
| 192.168.100.2 |  |  |  |  |
| 192.168.100.3 |  |  |  |  |
| 192.168.100.4 | Samsung Display 1 | Switch 2 | Yes | 45 |
| 192.168.100.5 | Samsung Display 2 | Switch 2 | Yes | 46 |
| 192.168.100.6 | Samsung Display 3 | Switch 2 | Yes | 47 |
| 192.168.100.7 | Samsung Display 4 | Switch 2 | Yes | 48 |
| 192.168.100.8 |  |  |  |  |
| 192.168.100.9 |  |  |  |  |
| 192.168.100.10 |  |  |  |  |
| 192.168.100.11 |  |  |  |  |
| 192.168.100.12 |  |  |  |  |
| 192.168.100.13 |  |  |  |  |
| 192.168.100.14 |  |  |  |  |
| 192.168.100.15 |  |  |  |  |
| 192.168.100.16 | Primary MediaServer | Switch 2 | Yes | 41 |
| 192.168.100.17 |  |  |  |  |
| 192.168.100.18 |  |  |  |  |
| 192.168.100.19 |  |  |  |  |
| 192.168.100.20 |  |  |  |  |
| 192.168.100.21 | Secondary MediaServer | Switch 2 | Yes | 42 |
| 192.168.100.22 |  |  |  |  |
| 192.168.100.23 |  | Switch 2 | Yes | 44 |
| 192.168.100.24 | Intel NUC PC AV Matrix Switcher | Switch 2 | Yes | 43 |
| 192.168.100.25 |  |  |  |  |
| 192.168.100.26 |  |  |  |  |
| 192.168.100.27 |  |  |  |  |
| 192.168.100.28 |  |  |  |  |
| 192.168.100.29 |  |  |  |  |
| 192.168.100.30 |  |  |  |  |
| 192.168.100.31 |  |  |  |  |
| 192.168.100.32 |  |  |  |  |
| 192.168.100.33 |  |  |  |  |
| 192.168.100.34 |  |  |  |  |
| 192.168.100.35 |  |  |  |  |
| 192.168.100.36 |  |  |  |  |
| 192.168.100.37 |  |  |  |  |
| 192.168.100.38 |  |  |  |  |
| 192.168.100.39 | PICOCELL | Switch 2 |  | 39 |
| 192.168.100.40 | PICOCELL | Switch 2 |  | (Ge1) 2/1/2 |
| 192.168.100.41 |  |  |  |  |
| 192.168.100.42 |  |  |  |  |
| 192.168.100.43 |  |  |  |  |
| 192.168.100.44 |  |  |  |  |
| 192.168.100.45 |  |  |  |  |
| 192.168.100.46 |  |  |  |  |
| 192.168.100.47 |  |  |  |  |
| 192.168.100.48 |  |  |  |  |
| 192.168.100.49 |  |  |  |  |
| 192.168.100.50 |  |  |  |  |
| 192.168.100.51 |  |  |  |  |
| 192.168.100.52 |  |  |  |  |
| 192.168.100.53 |  |  |  |  |
| 192.168.100.54 |  |  |  |  |
| 192.168.100.55 |  |  |  |  |
| 192.168.100.56 |  |  |  |  |
| 192.168.100.57 | RFID POC | Switch 2 |  | 46 |
| 192.168.100.58 | RFID POC | Switch 2 |  | 47 |
| 192.168.100.59 | RFID POC | Switch 2 |  | 48 |
| 192.168.100.60 | RetailNext | Switch 2 |  | 35 |
| 192.168.100.61 | Azure Stack Edge | Switch 2 | Yes | 48 |
| 192.168.100.62 | Azure Stack Edge | Switch 2 | Yes | 48 |
| 192.168.100.63 | Azure Stack Edge | Switch 2 | Yes | 48 |

| IP Address Device / Application Router / Switch POC List Port |  |  |  |  |
| --- | --- | --- | --- | --- |
| 192.168.120.1 | Guest Access (Router) | Rotuer |  | N/A |
| 192.168.120.2 |  |  |  |  |
| 192.168.120.3 |  |  |  |  |
| 192.168.120.4 |  |  |  |  |
| 192.168.120.5 |  |  |  |  |
| 192.168.120.6 |  |  |  |  |
| 192.168.120.7 |  |  |  |  |
| 192.168.120.8 |  |  |  |  |
| 192.168.120.9 |  |  |  |  |
| 192.168.120.10 |  |  |  |  |
| 192.168.120.11 |  |  |  |  |
| 192.168.120.12 |  |  |  |  |
| 192.168.120.13 |  |  |  |  |
| 192.168.120.14 |  |  |  |  |
| 192.168.120.15 |  |  |  |  |
| 192.168.120.16 |  |  |  |  |
| 192.168.120.17 |  |  |  |  |
| 192.168.120.18 |  |  |  |  |
| 192.168.120.19 |  |  |  |  |
| 192.168.120.20 |  |  |  |  |
| 192.168.120.21 |  |  |  |  |
| 192.168.120.22 |  |  |  |  |
| 192.168.120.23 |  |  |  |  |
| 192.168.120.24 |  |  |  |  |
| 192.168.120.25 |  |  |  |  |
| 192.168.120.26 |  |  |  |  |
| 192.168.120.27 |  |  |  |  |
| 192.168.120.28 |  |  |  |  |
| 192.168.120.29 |  |  |  |  |
| 192.168.120.30 |  |  |  |  |
| 192.168.120.31 |  |  |  |  |
| 192.168.120.32 |  |  |  |  |
| 192.168.120.33 |  |  |  |  |
| 192.168.120.34 |  |  |  |  |
| 192.168.120.35 |  |  |  |  |
| 192.168.120.36 |  |  |  |  |
| 192.168.120.37 |  |  |  |  |
| 192.168.120.38 |  |  |  |  |
| 192.168.120.39 |  |  |  |  |
| 192.168.120.40 |  |  |  |  |
| 192.168.120.41 |  |  |  |  |
| 192.168.120.42 |  |  |  |  |
| 192.168.120.43 |  |  |  |  |
| 192.168.120.44 |  |  |  |  |
| 192.168.120.45 |  |  |  |  |
| 192.168.120.46 |  |  |  |  |
| 192.168.120.47 |  |  |  |  |
| 192.168.120.48 |  |  |  |  |
| 192.168.120.49 |  |  |  |  |
| 192.168.120.50 |  |  |  |  |
| 192.168.120.51 |  |  |  |  |
| 192.168.120.52 |  |  |  |  |
| 192.168.120.53 |  |  |  |  |
| 192.168.120.54 |  |  |  |  |
| 192.168.120.55 |  |  |  |  |
| 192.168.120.56 |  |  |  |  |
| 192.168.120.57 |  |  |  |  |
| 192.168.120.58 |  |  |  |  |
| 192.168.120.59 |  |  |  |  |
| 192.168.120.60 |  |  |  |  |
| 192.168.120.61 |  |  |  |  |
| 192.168.120.62 |  |  |  |  |
| 192.168.120.63 |  |  |  |  |

| IP Address Device / Application Router / Switch VoIP - Vlan 50 |  |  |  |
| --- | --- | --- | --- |
| 10.x.y.193 | Interface VLAN 50 (Router) | Rotuer | Port |
| 10.x.y.194 | BCM50 Phone System | Switch 1 | N/A |
|  |  |  | 11 |
| 10.x.y.195 |  |  |  |

SAP / Port 41 42 43 44 45 46 47 436 Primary MediaServer Secondary MediaServer AV Matrix Switcher Samsung Display 1 Samsung Display 2 Samsung Display 3 Cisco 9300 - switch

| 737E 896E |  |  |  | Intel NUC PC |  |  |  | 48 Samsung Display 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 908E 9251 | Customer Welcome DisplayCustomer Welcome Display |  |  |  |  |  |  |  |
| 8894 9101 | PICO CELL 2 |  |  |  | RetailNext |  |  |  |
| 1294 9960 |  |  |  |  | RetailNext RetailNext |  |  |  |
| 7510 5843 |  |  |  |  | RetailNext RetailNext |  |  |  |
|  |  |  |  |  | RetailNext RetailNext |  |  |  |
| 9447 9926 |  |  |  |  |  |  |  |  |
| 9453 1099 |  |  |  |  |  |  |  |  |
| 903C 738E |  |  |  |  |  |  |  |  |
| 8030 8618 |  |  |  |  |  |  |  |  |
| 4712 4235 |  |  |  |  |  |  |  |  |
| 206F 1302 |  |  |  |  |  |  |  |  |
| 9769 213 |  |  |  |  |  |  |  |  |
| 9482 4445 |  |  |  |  |  |  |  |  |
| 7691 4679 |  |  |  |  |  |  |  |  |
| 8136 1064 |  |  |  |  |  |  |  |  |
| 8754 8012 |  |  |  |  |  |  |  |  |
| 90564020 5310 5891 |  |  |  |  |  | RFID POC RFID POC | RFID POC RFID POC |  |
|  |  |  |  |  |  |  |  | RFID POC RFID POC |

| Version | Comment | Date |
| --- | --- | --- |
| 1.0.0 | Initial Version | 4/10/2024 |
| 1.0.1 | Added PDU port-map | 9/19/2024 |